# Infonique Sound Recognition

| Prepared by | Date | Version |
| --- | --- | --- |
| Bing Ran | 03/20/2025 | 1.0 |

# 1 Abstract

This document details the implementation of a sound recognition system using the ESP32, covering both hardware and software aspects. The system is designed to recognize voice commands, process them, and respond with spoken feedback using pre-recorded audio files. The hardware includes an ESP32 Dev module, a VC-02 voice recognition module, an I2S audio output system, and an OLED display for visual feedback. The software integrates voice detection, serial communication, and audio playback . This document provides an in-depth exploration of system design, firmware development, and practical considerations, offering a comprehensive guide for developers interested in embedded sound recognition applications.

# Document History

| Date | Rev | Modifier | Changes |
| --- | --- | --- | --- |
| 20-March-2025 | 1.0 | Bing Ran | First Draft |

# Contents

Abstract	2

Document History	2

Contents	3

Table of Figures	4

Table of table	4

1 Introduction	5

2 Drone main board	5

2.1 Drone Main Board Pinout	6

2.2 Connectors	7

2.2.1 Power input connector	7

2.2.2 Motor	7

2.2.3 Motor control connectors	8

2.2.4 SD card connector	8

2.2.5 Barometer BME/BMP 280 module	9

2.2.6 Accelerator & gyro sensor MPU6050 module	9

2.2.6 GPS GT-U7 module	10

GT-U7 module able to communicate through UART communication	10

2.2.7 Radio frequency nRF24L01+ 2.4GHz wireless RF transceiver module	10

RF module able to communicate through SPI communication	10

3 Drone controller board	12

3.1 Pinout	12

# Table of Figures

Figure 1: Top and bottom layer of drone main board	5

Figure 2: Battery Connector	7

Figure 3: Motor power connectors	7

Figure 4: Motor control connectors	8

Figure 5: SD card connector & module	8

Figure 6: BME/BMP280 connector & module	9

Figure 7: MPU6050 connector & module	9

Figure 8: GPS GT-U7 connector & module	10

Figure 9: RF module connectors & module	10

Figure 10: Ultrasonic sensor connectos & module	11

Figure 11: Bluetooth connector & module	11

# Table of table

Table 1  Expansion board pinout	5

# 1 Introduction

## 1.1 Overview of Sound Recognition Technology

Sound recognition technology allows electronic devices to interpret and respond to audio signals, enabling voice commands, environmental sound detection, and real-time interaction. This technology is widely used in applications such as voice assistants (e.g., Alexa, Google Assistant), security systems, and accessibility tools for individuals with disabilities.

At its core, sound recognition involves capturing audio signals, processing them to extract features, and matching them to predefined patterns. With the advancement of machine learning and edge computing, microcontrollers like the ESP32 can now perform sound recognition efficiently without requiring cloud-based processing.

## 1.2 Importance of Embedded Sound Recognition

Embedded sound recognition plays a crucial role in creating smart and interactive systems. Unlike cloud-based solutions, embedded sound recognition processes audio locally on the device, offering advantages such as:

- Faster response time (no internet dependency)

- Better privacy and security (audio data is not sent to external servers)

- Lower power consumption (suitable for battery-powered devices)

For students, learning embedded sound recognition provides valuable insights into digital signal processing (DSP), machine learning on microcontrollers, and real-world embedded system development.

### 1.3 Objectives of This Project

The goal of this project is to provide students with hands-on experience in implementing a sound recognition system using the ESP32 microcontroller. This system will:

- Recognize voice commands using the VC-02 voice module

- Retrieve and announce temperature & humidity data from an SHT31 sensor

- Provide audio responses via I2S speaker output

- Display real-time readings on an SSD1306 OLED screen

- Use LittleFS for storing and playing pre-recorded audio files

By completing this project, students will learn about microcontroller programming, interfacing with hardware components, and implementing embedded sound recognition in practical applications.

# 2. Hardware Selection

The selection of hardware components plays a crucial role in ensuring smooth operation and performance of the embedded sound recognition system. Each component has been chosen to provide efficient processing, reliable voice recognition, clear audio output, real-time display feedback, and accurate environmental sensing.

## 2.1 ESP32 – Microcontroller for Processing

Figure 1: ESP32 Dev Module Type C

The ESP32 is the core processing unit of this project, responsible for handling all data processing, communication, and control operations. It is chosen due to its:

- Dual-core processor for handling multiple tasks efficiently

- Wi-Fi & Bluetooth capabilities (though not used in this project, they allow future expansion)

- Integrated I2S, I2C, and UART interfaces for seamless connection with peripherals

- Low-power operation suitable for battery-powered applications

The ESP32 manages the voice recognition module (VC-02), processes temperature and humidity sensor readings (SHT31), controls the OLED display (SSD1306), and plays audio responses using the MAX98357A I2S audio amplifier.

| Component | ESP32 Pin | Function |
| --- | --- | --- |
| VC-02 TX | GPIO16 | UART RX (Receive voice commands) |
| VC-02 RX | GPIO17 | UART TX (Send data if required) |
| I2S BCLK (MAX98357A) | GPIO25 | I2S Bit Clock |
| I2S LRC (MAX98357A) | GPIO26 | I2S Left-Right Clock |
| I2S DOUT (MAX98357A) | GPIO27 | I2S Data Output |
| SSD1306 OLED SDA | GPIO21 | I2C Data |
| SSD1306 OLED SCL | GPIO22 | I2C Clock |
| SHT31 SDA | GPIO21 | I2C Data (Shared with OLED) |
| SHT31 SCL | GPIO22 | I2C Clock (Shared with OLED) |

Table 1: ESP32 Pin Connection

## 2.2 VC-02 Voice Recognition Module – Recognizing Voice Commands

Figure 2: VC-02 Module

The VC-02 module is a compact, pre-trained voice recognition module capable of recognizing custom voice commands. It is selected due to:

- Built-in offline voice recognition (no need for internet access)

- Customizable wake words & commands

- UART interface for easy communication with ESP32

- Low power consumption

### Function in the Project

- Detects specific voice commands like "Hi Pudding", "Humidity", “Temperature” and "Siren"

- Sends recognized command data to ESP32 via UART

2.3 I2S Audio Output (MAX98357A + Speaker) – Playing Responses

Figure 3: MAX98357A

The MAX98357A is a digital-to-analog converter (DAC) and audio amplifier that converts digital I2S signals into analog audio output, which is played through a connected speaker.

### Reasons for Choosing MAX98357A

- I2S support for high-quality audio playback

- Built-in amplifier eliminates the need for additional circuitry

- Compact and low-power design

### Function in the Project

- Receives pre-recorded WAV audio files stored in ESP32's LittleFS storage

- Converts digital audio signals into analog signals

- Outputs sound through a connected speaker

## 2.4 SSD1306 OLED Display – Visual Feedback

Figure 4: SSD1306 OLED Display

The SSD1306 OLED display is a 128x64 pixel monochrome display used to show real-time sensor readings and system status.

### Reasons for Choosing SSD1306

- Low power consumption (suitable for embedded applications)

- I2C communication (only requires 2 pins, SDA & SCL)

- Clear, high-contrast display for easy readability

### Function in the Project

- Displays real-time temperature and humidity readings

- Provides visual confirmation of recognized voice commands

## 2.5 SHT31 Temperature & Humidity Sensor – Sensor Data Acquisition

The SHT31 is a highly accurate temperature and humidity sensor that communicates with the ESP32 over I2C.

### Reasons for Choosing SHT31

- High accuracy (±0.3°C for temperature, ±2% for humidity)

- Fast response time

- I2C communication (easy interface with ESP32)

### Function in the Project

- Measures temperature and humidity in real-time

- Sends data to the ESP32 for processing and display

- Enables the audio response system to announce current weather conditions